TUTORIAL 1

- Explain, with a labelled diagram, the pinhole camera model. Derive the relationship between object distance, image distance, and focal length for a thin lens.

- Differentiate between rigid and affine transformations. Given a 2D point (3, 4), find its new coordinates after a rotation of 30° about the origin followed by a translation of (2, −1).

- What is meant by digitization of an image? Explain sampling and quantization with the help of a suitable diagram, and state what determines the spatial and grayscale resolution of a digital image.

- A grayscale image has pixel intensities concentrated mostly in the range 50–120 out of a possible 0–255. Explain what problem this causes visually, and describe step-by-step how histogram equalization would correct it.

- Differentiate between a low-pass and a high-pass spatial filter with one example kernel (3×3) each. State one practical use case for each.

- Perform 2D convolution of the 3×3 image patch below with the given 3×3 averaging kernel (show your working):

- Image patch: [[10,20,30],[40,50,60],[70,80,90]]

- Kernel: (1/9) × [[1,1,1],[1,1,1],[1,1,1]]

- Compute only the output value at the centre pixel.

- State the Fourier transform pair for a 2D image. Explain, conceptually, why a sharp edge in the spatial domain corresponds to high-frequency content in the frequency domain.

- Explain how Fourier-domain filtering can be used to remove periodic noise from an image that spatial filtering struggles with. Illustrate with a rough sketch of the magnitude spectrum before and after filtering.

- Short answers (1–2 lines each): (a) What is radiometry? (b) Define a colour transform with one example. (c) What is the difference between an affine and a projective transformation?  [6]