TUTORIAL 3

- Write the general form of the camera projection equation relating a 3D world point to its 2D image coordinates, and identify the intrinsic and extrinsic matrices within it.

- List the parameters typically contained in the intrinsic parameter matrix of a camera. What does each one physically represent?

- Define extrinsic camera parameters. Given a camera with rotation matrix R and translation vector t, explain what these represent physically for a camera mounted on a robot arm.

- Explain radial lens distortion. Sketch the difference between "barrel" and "pincushion" distortion, and state which is more common in wide-angle lenses.

- Describe, step by step, how a checkerboard pattern is used for direct camera parameter calibration (Zhang's method or equivalent covered in class).

- Given a camera projection matrix P (3×4), explain how the intrinsic and extrinsic parameters can be recovered from it using QR/RQ decomposition (conceptual explanation is sufficient, no need for full derivation).

- Compare orthographic, weak-perspective, and full-perspective camera models. For each, state one real scenario (e.g., satellite imaging, overhead warehouse camera, handheld phone camera close to a subject) where that model is a reasonable approximation.

- What is meant by reprojection error in the context of camera calibration? Why is it used as the main metric to judge calibration quality?

- A calibration run using 15 checkerboard images gives a lower reprojection error than one using only 5 images. Explain, in terms of the underlying optimization, why more images generally improve calibration accuracy.

- Short answers (1–2 lines each): (a) What is the principal point of a camera? (b) Define focal length in pixel units. (c) Why can't a single image alone be used to calibrate a camera?